# NETWORK SECURITY, MAINTENANCE AND TROUBLESHOOTING PROCESS

# DAY 1-5

TASK1: Assign static IP addresses to five devices (laptops or desktops) on your home or lab network, ensuring each is in the same subnet, and verify connectivity between them using the ping command.

- Connect all five devices to the same Wi-Fi router or switch.

- On each device, open Control Panel → Network and Sharing Center → Change Adapter Settings.

- Right-click Ethernet or Wi-Fi → Properties.

- Select Internet Protocol Version 4 (TCP/IPv4) and click Properties.

- Select Use the following IP address.

- Assign the following IP addresses:

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Laptop 1 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| Laptop 2 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| Laptop 3 | 192.168.1.12 | 255.255.255.0 | 192.168.1.1 |
| Desktop 1 | 192.168.1.13 | 255.255.255.0 | 192.168.1.1 |
| Desktop 2 | 192.168.1.14 | 255.255.255.0 | 192.168.1.1 |

7.Click OK and save the settings.

8.Open Command Prompt on one device.

9.Run: ping 192.168.1.11

10.Repeat for the other devices.

TASK2: Set up a DHCP server (using your router or software like Packet Tracer/GNS3/VirtualBox) to automatically assign IP addresses to at least three devices, and confirm that each device receives a unique IP from the correct range.

- Open Cisco Packet Tracer.

- Add one Server, one Switch, and three PCs.

- Connect all devices using Copper Straight-Through cables.

- Click the Server.

- Go to Services → DHCP.

- Turn DHCP Service ON.

- Configure:

  Pool Name: LAN

  Default Gateway: 192.168.1.1

  DNS Server: 8.8.8.8

  Start IP Address: 192.168.1.100

  Subnet Mask: 255.255.255.0

  Maximum Users: 10

- Click Save.

- On each PC, go to Desktop → IP Configuration.

- Select DHCP.

- Verify that each PC receives a different IP address.

EXAMPLE:

| Device | Assigned IP |
| --- | --- |
| PC0 | 192.168.1.100 |
| PC1 | 192.168.1.101 |
| PC2 | 192.168.1.102 |

TASK3: Configure a DNS service (using your router’s DNS settings or a tool like dnsmasq) so that typing a custom hostname (e.g., musicplayer.local) in a browser on any connected device opens a specific local web page or device interface.<br><br><em><strong>Hint:</strong> You can map hostnames to local IPs using the DNS service or by editing the hosts file for a quick test.</em>

- Open your router settings or dnsmasq.

- Add a new DNS record.

- Enter the following:

  Hostname: musicplayer.local

  IP Address: 192.168.1.50

- Save the configuration.

- Restart the DNS service if required.

- On another device connected to the same network, open a browser.

- Type: http://musicplayer.local

- Verify that the local webpage or device interface opens.

TASK4: Create two VLANs (e.g., VLAN 10: 'Workstations', VLAN 20: 'Smart Devices') on your switch or simulation tool, assign at least two devices to each VLAN, and demonstrate that devices in the same VLAN can communicate, but not with devices in the other VLAN.<br><br><em><strong>Hint:</strong> Use ping or file sharing to test connectivity between devices in different VLANs.</em>

- Open Cisco Packet Tracer.

- Add one 2960 Switch and four PCs.

- Connect all PCs to the switch.

- Open the switch CLI.

- Create VLAN 10:

enable

configure terminal

vlan 10

name Workstations

exit

6.Create VLAN 20:

vlan 20

name SmartDevices

exit

7.Assign ports:

interface range fa0/1-2

switchport mode access

switchport access vlan 10

exit

interface range fa0/3-4

switchport mode access

switchport access vlan 20

exit

8.Assign IP addresses:

i. VLAN 10 PCs:

- 192.168.10.2

- 192.168.10.3

ii.VLAN 20 PCs:

- 192.168.20.2

- 192.168.20.3

9.Ping between PCs in the same VLAN.

10.Ping between PCs in different VLANs.

TASK5: Simulate internet access for your LAN by configuring a router with NAT (Network Address Translation), then test that devices on your LAN can access an external site (e.g., www.wikipedia.org) or a simulated external server.

- Open Cisco Packet Tracer.

- Add:

  One Router

  One Switch

  Two PCs

  One Cloud (Internet)

- Connect all devices.

- Configure the LAN interface:

IP Address: 192.168.1.1

Subnet Mask: 255.255.255.0

- Configure the WAN interface:

IP Address: 203.0.113.2

Subnet Mask: 255.255.255.252

- Enable NAT on the router.

- Configure the PCs to use:

- Gateway: 192.168.1.1

- Test internet connectivity by opening a browser or using:

- Verify that the request is successful.