# TROUBLESHOOTING AND HELPDESK DAY2

TASK1: List 5 common hardware issues you might encounter in a desktop PC and write the typical symptom for each (for example: 'No display on monitor' — symptom: blank screen, no POST beep).

| Hardware Issue | Typical Symptom |
| --- | --- |
| No display on monitor | Blank screen, no POST beep. |
| Faulty RAM | PC beeps continuously or restarts repeatedly. |
| Hard disk failure | Operating system does not boot or "No Boot Device" error appears. |
| Faulty Power Supply (SMPS) | PC does not turn on or shuts down unexpectedly. |
| CPU Overheating | PC becomes very hot and shuts down automatically. |

TASK2: Given this scenario: A laptop is not charging even when plugged in. Write down a step-by-step troubleshooting plan using only basic tools (no software diagnostics), and explain what you would check at each step.

Step 1: Check the Power Outlet

- Plug another device into the same socket.

- Check: Is the power outlet working?

Step 2: Inspect the Charger

- Look for cuts, burns, or loose connections.

- Check: Is the charger physically damaged?

Step 3: Check the Charging Port

- Inspect the laptop charging port.

- Check: Is the port loose, dirty, or damaged?

Step 4: Check the Charging Indicator

- Connect the charger.

- Check: Does the charging LED turn on?

Step 5: Test with Another Charger

- Use another compatible charger.

- Check: Does the laptop start charging?

Step 6: Check the Battery

- If possible, remove and reconnect the battery (for removable batteries).

- Check: Is the battery properly connected?

TASK3: Use a multimeter (real or virtual simulator) to test the voltage output of a standard USB phone charger. Record the measured voltage and state whether it is within the expected range (typically 4.75V to 5.25V for USB).

- Set the multimeter to DC Voltage (20V range).

- Plug the USB phone charger into a power outlet.

- Identify the +5V and Ground (GND) pins of the USB connector.

- Place the black probe of the multimeter on the Ground (GND) pin.

- Place the red probe on the +5V pin.

- Read the voltage displayed on the multimeter.

- Record the measured voltage.

- Compare the measured value with the standard USB voltage range

TASK4: A PC is randomly shutting down. Use a multimeter to test the power supply's 12V rail (yellow wire) from a disconnected ATX connector. Describe the steps and write down the voltage you find. If the voltage is below 11.5V, what hardware issue could this indicate?<br><br><em><strong>Hint:</strong> Remember to set the multimeter to DC voltage and probe between the yellow and black wires.</em

- Turn off the computer and disconnect the power cable.

- Disconnect the 24-pin ATX power connector from the motherboard.

- Set the multimeter to DC Voltage (20V range).

- Locate a yellow wire (+12V) and a black wire (Ground) on the ATX connector.

- Connect the black probe to the black (Ground) wire.

- Connect the red probe to the yellow (+12V) wire.

- Turn on the power supply .

- Read the voltage displayed on the multimeter.

- Record the measured voltage.

- Compare the measured value with the expected 12V output.