# TROUBLESHOOTING AND HELPDESK DAY4

TASK1: Open your desktop or laptop, locate the RAM slots, and remove and re-insert the RAM module to simulate reseating — then document each step you took and any issues you observed during the process.

- Shut down the computer.

- Disconnect the power cable and remove the battery (if using a laptop with a removable battery).

- Press the power button for a few seconds to discharge any remaining power.

- Open the computer case or laptop back cover.

- Locate the RAM slots on the motherboard.

- Push the clips on both sides of the RAM module to release it.

- Carefully remove the RAM module.

- Check for dust or dirt and clean gently if necessary.

- Reinsert the RAM module firmly into the slot until both clips lock automatically.

- Close the computer case.

- Reconnect the power and start the computer.

- Verify that the system boots normally.

TASK2: .Create a troubleshooting checklist for diagnosing a faulty GPU in a gaming PC, including at least 5 steps from initial symptom identification to confirming the fix.<br><br><em><strong>Hint:</strong> Think about display issues, driver checks, and physical connections.</em>

| Step | Action |
| --- | --- |
| 1 | Check if the monitor displays a blank screen, artifacts, or "No Signal." |
| 2 | Verify that the monitor cable is securely connected to the graphics card. |
| 3 | Remove and firmly reinsert the GPU into the PCIe slot. |
| 4 | Check that all GPU power connectors are properly connected. |
| 5 | Update or reinstall the graphics driver. |
| 6 | Test the GPU in another PC or test another GPU in the same PC. |
| 7 | Confirm that the display works correctly after the fix. |

TASK3: Simulate a storage issue by unplugging and replugging a SATA or NVMe drive (SSD/HDD) on a test system, then describe the error messages or BIOS indicators you see before and after reconnecting.

PROCEDURE:-

- Turn off the computer.

- Disconnect the power cable.

- Open the computer case.

- Disconnect the SATA cable from the HDD/SSD (or remove the NVMe SSD).

- Turn on the computer.

BEFORE RECONNECTING:-

- No Boot Device Found

- Operating System Not Found

- Boot Device Not Detected

- Storage drive not listed in BIOS

- Turn off the computer again.

- Reconnect the SATA cable or reinstall the NVMe SSD.

- Restart the computer.

AFTER RECONNECTING:-

- The BIOS detects the SSD/HDD.

- The operating system starts normally.

TASK4: Given a scenario where a PC randomly restarts, list which component (RAM, GPU, storage) you would check first and explain your reasoning, then outline the specific diagnostic steps you would perform for that component.

First I check RAM because RAM is one of the most common causes of random restarts. A loose or faulty RAM module can lead to system crashes, restart loops, or blue screen errors.

STEPS TO RESOLVE PROBLEM:-

- Turn off the computer and disconnect the power.

- Open the computer case.

- Remove the RAM module.

- Check for dust or damage.

- Reinsert the RAM securely.

- If there are multiple RAM modules, test one module at a time.

- Restart the computer and observe whether the restart issue is resolved.