# NETWORK FUNDAMENTALS AND BUILDING NETWORKS

IP ADDRESSING AND SUBNETTING

TASK1: List the IP address classes (A, B, C, D, E) and give one real-world example of a device or service that might use each class. Write your answers in a table format.

| Class | IP Address Range | Real-World Example |
| --- | --- | --- |
| Class A | 1.0.0.0 – 126.255.255.255 | Large companies like Google |
| Class B | 128.0.0.0 – 191.255.255.255 | University or college network |
| Class C | 192.0.0.0 – 223.255.255.255 | Home Wi-Fi router |
| Class D | 224.0.0.0 – 239.255.255.255 | Live video streaming (IPL streaming) |
| Class E | 240.0.0.0 – 255.255.255.255 | Research and testing |

TASK2: Given the IP address 192.168.10.45 with subnet mask 255.255.255.0, calculate the network address, broadcast address, and the range of valid host addresses.<br><br><em><strong>Hint:</strong> Show your working steps for each calculation.</em>

GIVEN:

- IP Address = 192.168.10.45

- Subnet Mask = 255.255.255.0 (/24)

### Step 1: Network Address

The last part becomes 0.

Network Address = 192.168.10.0

Step 2: Broadcast Address

The last part becomes 255.

Broadcast Address = 192.168.10.255

Step 3: Valid Host Range

First usable IP:

192.168.10.1

Last usable IP:

192.168.10.254

| Item | Answer |
| --- | --- |
| Network Address | 192.168.10.0 |
| Broadcast Address | 192.168.10.255 |
| First Host | 192.168.10.1 |
| Last Host | 192.168.10.254 |

TASK3: Create a subnetting plan for a small food delivery startup like Zomato that needs 4 separate subnets: one for Delivery Partners, one for Restaurants, one for Customers, and one for Admins. Assign a unique subnet (with network address and subnet mask) for each group using the 10.0.0.0/24 network.

| Group | Network Address | Subnet Mask |
| --- | --- | --- |
| Delivery Partners | 10.0.0.0/26 | 255.255.255.192 |
| Restaurants | 10.0.0.64/26 | 255.255.255.192 |
| Customers | 10.0.0.128/26 | 255.255.255.192 |
| Admins | 10.0.0.192/26 | 255.255.255.192 |

TASK4: You are given the following scenario: A college WiFi network uses the IP range 172.16.0.0/16. The admin wants to create at least 20 subnets for different buildings. Calculate the new subnet mask and the number of hosts available per subnet.<br><br><em><strong>Constraint:</strong> Show your calculation steps and final answers.</em>

GIVEN:

- Network = 172.16.0.0/16

- Required = 20 subnets

### Step 1

Borrow 5 bits because:

2⁵ = 32 subnets (enough for 20)

Step 2

Original subnet mask:

16 = 255.255.0.0

New subnet mask:

21 = 255.255.248.0

Step 3

Host bits left:

32 − 21 = 11 bits

Hosts per subnet:

2¹¹ − 2 = 2046 hosts

| Item | Value |
| --- | --- |
| New Subnet Mask | 255.255.248.0 (/21) |
| Number of Subnets | 32 |
| Hosts per Subnet | 2046 |

TASK5: A friend sends you a message asking why their device with IP 192.168.1.300 is not connecting to the network. Write a short explanation identifying the error and what a valid IP address in that subnet should look like.

The IP address 192.168.1.300 is not valid because each part (called an octet) must be between 0 and 255. The number 300 is outside this range, so the device cannot use this IP address.

A valid IP address in the same subnet could be:

- 192.168.1.100

- 192.168.1.150

- 192.168.1.200

These IP addresses are valid because all four numbers are between 0 and 255.