# NETWORK FUNDAMENTALS AND BUILDING NETWORKS DAY3

TASK1: List the key differences between IPv4 and IPv6 addressing by creating a two-column table with at least 4 points for each.

| IPv4 | IPv6 |
| --- | --- |
| Uses 32-bit addresses. | Uses 128-bit addresses. |
| Address format: 192.168.1.1 | Address format: 2001:db8::1 |
| Supports about 4.3 billion addresses. | Supports a very large number of addresses (almost unlimited). |
| Uses decimal numbers separated by dots. | Uses hexadecimal numbers separated by colons. |
| Example: 192.168.10.25 | Example: 2001:db8:85a3::8a2e:370:7334 |

TASK2: Given the following scenario: You have 4 devices (laptop, phone, smart TV, and printer) connected to your home WiFi. Assign a unique static IPv4 address to each device using the 192.168.10.0/24 network, and write down the IP, subnet mask, and default gateway for each device.

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Laptop | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| Phone | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 |
| Smart TV | 192.168.10.12 | 255.255.255.0 | 192.168.10.1 |
| Printer | 192.168.10.13 | 255.255.255.0 | 192.168.10.1 |

TASK3: Calculate the number of valid subnets and hosts per subnet if you use a /27 CIDR subnet mask on a 192.168.1.0 network. List the first three valid subnet address ranges (network address to broadcast address) and the usable host IPs in each.

GIVEN:

- Network: 192.168.1.0

- CIDR: /27

CALCULATION:

- Default subnet mask = /24

- New subnet mask = /27

- Borrowed bits = 3

- Number of subnets = 2³ = 8

- Host bits = 5

- Hosts per subnet = 2⁵ − 2 = 30 usable hosts

| Subnet | Network Address | Usable Host Range | Broadcast Address |
| --- | --- | --- | --- |
| 1 | 192.168.1.0 | 192.168.1.1 – 192.168.1.30 | 192.168.1.31 |
| 2 | 192.168.1.32 | 192.168.1.33 – 192.168.1.62 | 192.168.1.63 |
| 3 | 192.168.1.64 | 192.168.1.65 – 192.168.1.94 | 192.168.1.95 |

TASK4: You are given the IP address 10.0.5.17 with subnet mask 255.255.255.0. Determine the network address, broadcast address, and the range of valid host addresses for this subnet.<br><br><em><strong>Hint:</strong> Convert the subnet mask to CIDR notation and use bitwise AND for network address calculation.</em>

GIVEN:

- IP Address: 10.0.5.17

- Subnet Mask: 255.255.255.0

CALVULATION:

Step 1: Convert to CIDR

255.255.255.0 = /24

Step 2: Network Address

10.0.5.0

Step 3: Broadcast Address

10.0.5.255

Step 4: Valid Host Range

First Host: 10.0.5.1

Last Host: 10.0.5.254

| Item | Value |
| --- | --- |
| CIDR | /24 |
| Network Address | 10.0.5.0 |
| Broadcast Address | 10.0.5.255 |
| Valid Host Range | 10.0.5.1 – 10.0.5.254 |

TASK5: Use an online subnet calculator tool (such as https://www.subnet-calculator.com/) to verify your answer for the previous task. Take a screenshot of your calculation and write one line explaining if your manual answer matched the tool's output.

- Opened the website https://www.subnet-calculator.com/ in a web browser.

- Entered the IP address 10.0.5.17.

- Entered the subnet mask 255.255.255.0.

- Clicked Calculate.

- Compared the results with my manual calculation.

- Took a screenshot of the calculator output.

OBSERVATION:

The subnet calculator showed:

- Network Address: 10.0.5.0

- Broadcast Address: 10.0.5.255

- Host Range: 10.0.5.1 – 10.0.5.254

These values matched my manual calculation.