# NETWORK FUNDAMENTALS AND BUILDING NETWORKS DAY4

TASK1: Given a network address of 192.168.10.0/24, divide it into 4 equal subnets and write down the subnet mask, network address, and broadcast address for each subnet.

- Original Network: 192.168.10.0/24

- Required Subnets: 4

- Borrowed Bits = 2

- Subnet Mask = 255.255.255.192

- Hosts per Subnet = 62 usable hosts

| Subnet | Network Address | Subnet Mask | Broadcast Address |
| --- | --- | --- | --- |
| Subnet 1 | 192.168.10.0 | 255.255.255.192 (/26) | 192.168.10.63 |
| Subnet 2 | 192.168.10.64 | 255.255.255.192 (/26) | 192.168.10.127 |
| Subnet 3 | 192.168.10.128 | 255.255.255.192 (/26) | 192.168.10.191 |
| Subnet 4 | 192.168.10.192 | 255.255.255.192 (/26) | 192.168.10.255 |

TASK2: For one of the subnets you created, assign valid IP addresses to 5 devices (such as a router, two laptops, a printer, and a mobile) and list which IP goes to which device.<br><br><em><strong>Hint:</strong> Make sure not to use the network or broadcast address for any device.</em>

Selected Subnet

Subnet 1: 192.168.10.0/26

Usable IP Range: 192.168.10.1 – 192.168.10.62

| Device | Assigned IP Address |
| --- | --- |
| Router | 192.168.10.1 |
| Laptop 1 | 192.168.10.2 |
| Laptop 2 | 192.168.10.3 |
| Printer | 192.168.10.4 |
| Mobile Phone | 192.168.10.5 |

TASK3: Draw a simple diagram (hand-drawn or digital) showing your subnet plan: label each subnet, the range of IPs, and where each device is connected.

TASK4: Document step-by-step how you calculated the subnet mask, number of hosts per subnet, and assigned IP addresses for your plan, as if you are explaining it to a friend who is new to subnetting.

Steps Performed

Step 1: Started with the network 192.168.10.0/24.

Step 2: I needed 4 subnets, so I borrowed 2 bits from the host portion because 2² = 4 subnets.

Step 3: The subnet mask changed from /24 to /26, which is 255.255.255.192.

Step 4: Each subnet contains 64 IP addresses, but 2 addresses (network and broadcast) cannot be used. Therefore, each subnet has 62 usable host addresses.

Step 5: I listed the four subnet network addresses and broadcast addresses by increasing the network address by 64 each time:

- 192.168.10.0

- 192.168.10.64

- 192.168.10.128

- 192.168.10.192

Step 6: I selected the first subnet and assigned valid IP addresses to the router, laptops, printer, and mobile phone. I made sure not to use the network address or broadcast address.